# HMI Touch CR-CH-HP Upgrade Procedure

Note: to upgrade the AVD PLC You need the Software FREE STUDIO PLUS available from the web site www.eliwell.it. To download it register on the website and select the option FreeWay product.

To upgrade your controller AVP1000 with the lastest version please follow the next instruction:

- Upgrade the PLC BIOS following the next steps:

- Open Free Studio Plus Installer and open the project file HMI_CH_HP.CFN.

- Connect a miniUSB cable from Your laptop to the PLC AVP1000

- Check the connection setting and connect to the target.

-Check status and select bios Download

- Select last Bios and select Download

- Wait until the PLC reboot

- As soon the PLC reboot select the Select the Option download all.

If the controller is allready running in a real application, we recommend to the st the procedure in a backup controller to avoid any unconsidered issue in the upgrading procedure. This expecialy if it can have an impact in the “cold chain” in case application for food conservation.